For Stage 1: Handoff, implementation.

## Vivado

- Create blank project

- Create Block Design

- Add MPSoC IP

  - Run block automation – apply board presets

- Tie both fpd_aclk to pl_clk0

- Validate Design

- Create HDL Wrapper

  - Let Vivado manage

- Generate Bitstream

- Export Hardware

  - Include Bitstream

## Vitis

- Create Platform Component

  - Hardware Design – From the .XSA exported in Vivado step 8

    - OS - Standalone

    - Processor – psu_cortexa53_0

    - Generate Boot Artifacts

    - Generate PMU Firmware

    - Target FSBL – psu_cortexr5_0

  - Build Platform

- Create FSBL

  - New Component > From Example

  - Zynq MP FSBL

    - Create new domain

    - OS – Standalone

    - Processor – psu_cortexr5_0

- Create Test Application

  - New Component > From Example

  - Hello World

    - Select new Cortex R5 standalone domain

  - Select application in Components pane

  - Sources > src > lscript.ld

    - Set all memory region fields to psu_r5_0_atcm_MEM_0

- Builds

  - Rebuild Platform

  - Build FSBL

  - Build Hello_world

- Create Boot Image

  - Select hello_world in components pane

  - Select Create Boot Image in flow pane

  - Remove all entries

  - Add .elfs – located in each item’s ‘build’ folder

    - Hello_world.elf

      - Type - Datafile

      - Destination – r5_0

    - Fsbl.elf

      - Type - Bootloader

      - Destination – r5_0

    - Pmufw.elf

      - Located in ./platform/export/platform/sw/boot

      - Type – pmu

    - .bit

      - ./platform/export/platform/hw/sdt/*.bit

  - Reorder items in list

    - FSBL > PMUFW > .bit > hello_world

    - Type – datafile

    - Destination device – pl

    - Destination cpu – r5_0