For Stage 1: Handoff, implementation.

## Vivado

- Create blank project

- Create Block Design

- Add MPSoC IP

  - Run block automation – apply board presets

- Tie both fpd_aclk to pl_clk0

- Validate Design

- Create HDL Wrapper

  - Let Vivado manage

- Generate Bitstream

- Export Hardware

  - Include Bitstream

## Vitis

- Create Platform Component

  - Hardware Design – From the .XSA exported in Vivado step 8

    - OS - Standalone

    - Processor – psu_cortexa53_0

    - Generate Boot Artifacts

    - Generate PMU Firmware

    - Target FSBL – psu_cortexr5_0

  - Build Platform

- Create FSBL

  - New Component > From Example

  - ZynqMP FSBL

    - Create new domain

    - OS – Standalone

    - Processor – psu_cortexr5_0

- Create PMUFW

  - New Component > From Example

  - ZynqMP PMU Firmware

    - OS – Standalone

    - Processor – psu_pmu_0

- Create Test Application

  - New Component > From Example

  - Hello World

    - Select new Cortex R5 standalone domain

  - Select application in Components pane

  - Sources > src > lscript.ld

    - Set all memory region fields to psu_r5_0_atcm_MEM_0

- Builds

  - Rebuild Platform

  - Build FSBL

  - Build PMUFW

  - Build Hello_world

- Create Boot Image (note: existing .bif can be imported if this step is being repeated)

  - From top menu bar

    - Vitis > Create Boot Image > Zynq Ultrascale+

  - Remove all entries

  - Add .elfs – located in each item’s ‘build’ folder

    - Hello_world.elf

      - Type - Datafile

      - Destination – r5_0

    - Fsbl.elf

      - Type - Bootloader

      - Destination – r5_0

    - Pmufw.elf

      - Type – datafile

      - Destination – pmu

    - .bit

      - Platform\export\platform\hw\sdt\*.bit

      - Type – datafile

      - Destination device - PL

  - Reorder items in list

    - FSBL > PMUFW > .bit > hello_world

  - Create BOOT.bin

- Boot from SD card

  - Set boot switches to off-on-off-on

  - Place BOOT.bin on SD and insert into board

  - Boot board with SW7. This will execute program.

  - To verify, connect to board over UART, restart board with SW6 to view output

## Phase 2: Attaching Keys and updating PMUFW.

The authors do not use an unmodified PMUFW template. The paper refers to the Secure Processor Block as the base root of trust, stating that it contains hardcoded keys. In the code, this is contained within the PMUFW and executes on the pmu.

- Generate AES key for bitstream encryption

  - Create a .nky file with these contents:

Device xczu1cg; Key 0 <PASTE_YOUR_64_CHAR_KEY_HERE>; IV 0 <PASTE_YOUR_24_CHAR_IV_HERE>;

  - Create and replace 64_char_key with output of:
openssl rand -hex 32

  - Create and replace 24_char_IV with
openssl rand -hex 12

- Generate RSA-4096 Pair

  - openssl genrsa -out device_key_4096.pem 4096

  - openssl rsa -in device_key_4096.pem -text -noout > key_dump.txt

- Copy U96 pmufw custom files into our pmufw src files

  - xpfw_mod_sec.c

  - xpfw_mod_sec.h

- Modify xpfw_mod_sec.c with keys from key_dump.txt

  - For these, convert “<hex>:<hex>” to “0x<hex>,0x<hex>”

  - Replace root_sk with privateExponent

  - Replace root_mod with modulus

  - Leave root_pk untouched

  - If first hex value of modulus is 0x00, key may have been padded by openssl. check number of hex values in key. If total is 513, delete the first 0x00 value.

- Modify xpfw_mod_sec.c further

  - #include xfpga_config.h is old vitis

    - comment out

    - replace with #include “xilfpga.h”

  - #define XPAR_XCSUDMA_0_DEVICE_ID 0

  - Implement sec_load_bistream() fix

- Modify xpfw_user_startup.c

  - #include "xpfw_mod_sec.h"

  - sec_ipi_mod_init();

    - make that the final line in the file

- Rebuild pmufw code, create new boot.bin, verify expected print statements from putty.

## Phase 3: FSBL Modification

The authors do not use an unmodified FSBL template. Their custom FSBL implements the hashing engine.

- Add dev_key.h to our FSBL

  - Copy this file over from the u96 FSBL code.

  - Contents of the arrays need to be corrected

    - Copy in root_sk and root_mod from pmufw/xpfw_mod_sec.c

- Modify xfsbl_hw.h

  - place after XFSBL_OCM_END_ADDRESS definition

  - Add OCM_SEC_BUFFER_ADDRESS definition

    - #define OCM_SEC_BUFFER_ADDRESS		(0xFFFFFE00)

    - The above address might cause issues. It lies within the ‘scratchpad’ memory, in the final 512 bytes. It is possible that another FSBL operation will overwrite this memory location. Potential source of silent error.

  - Add SEC_BUFFER_SIZE definition

    - #define SEC_BUFFER_SIZE 512

- Modify xfsbl_main.c

  - Add includes:

    - #include “xsecure_sha.h”

    - #include “dev_key.h”

    - #include "xfsbl_authentication.h"

  - Global var:

    - Place after XFsblPs FsblInstance = {0x3U, …

    - static XSecure_Sha3 csu_sha3;

  - Add major ShEF segment

    - Place at end of XFSBL_STAGE3 segment after

FsblStage = XFSBL_STAGE4; EarlyHandoff = FsblStatus;

    - NOTE: This code contains partition indices that will need to be defined later. Marked by TODO:

    - Custom ShEF code block

## Phase 4: Security Kernel Implementation

Enter at main()
Relies on keys stored at 0xFFFFFE00 by fsbl. 
reads those keys and uses ed25519 to build key-pair attest_pk and attest_sk
uses kernel_hash and attest_pk to get sha-3 hash.
sends hash to pmu
PMU signs hash and returns signature
enters loop and waits for handshake.
once channel established kernel decrypts bitstream and loads it to PL

### sec_load_bitstream() fix

the sec_load_bistream() function in the u96 version of xpfw_mod_sec.c is incompatible with modern vitis. Replace that function entirely with this version:

static void sec_load_bitstream(){ u32 status; s32 fpga_status; u8 bitstream_digest[48]; u32 msg_buf[8]; u32 resp_buf[2] = {0}; u32 bytes_sent = 0; // NEW: Structure for 2023.2 XilFPGA API XFpga XFpgaInstance = {0}; if(bitstream_size == 0 ¦¦ bitstream_addr == NULL){ XPfw_Printf(DEBUG_ERROR, "PMU: Bitstream address or size error\r\n"); return; } // 1. Hash the bitstream XCsuDma_Config *csu_config; csu_config = XCsuDma_LookupConfig(XPAR_XCSUDMA_0_DEVICE_ID); if (csu_config == NULL){ XPfw_Printf(DEBUG_ERROR, "PMU: Failed to configure CSU\r\n"); return; } status = XCsuDma_CfgInitialize(&csu_dma, csu_config, csu_config->BaseAddress); if (status != XST_SUCCESS){ XPfw_Printf(DEBUG_ERROR, "PMU: Failed to initialize CSU\r\n"); return; } XSecure_Sha3Initialize(&secure_sha3, &csu_dma); // Cast to (u8*) to fix volatile warning XSecure_Sha3Digest(&secure_sha3, (u8*)bitstream_addr, bitstream_size, bitstream_digest); // 2. Load the Bitstream (FIXED FOR 2023.2 API) // Initialize the XilFPGA library fpga_status = XFpga_Initialize(&XFpgaInstance); if (fpga_status != XST_SUCCESS) { XPfw_Printf(DEBUG_ERROR, "PMU: XFpga Init Failed\r\n"); return; } /* * API: XFpga_BitStream_Load(InstancePtr, Addr, KeyAddr, Size, Flags) * KeyAddr = 0 (We are not using an external user key here) * Flags = 0 (Implies XFPGA_FULL_BITSTREAM) */ fpga_status = XFpga_BitStream_Load(&XFpgaInstance, (UINTPTR)bitstream_addr, 0, bitstream_size, 0); if(fpga_status == XFPGA_SUCCESS){ XPfw_Printf(DEBUG_DETAILED, "PMU: PL Configuration successful\r\n"); } else{ XPfw_Printf(DEBUG_DETAILED, "PMU: PL Configuration failed %d\r\n", fpga_status); } // 3. Send Response while(bytes_sent < SHA3_SIZE){ msg_buf[1] = IPI_BITSTREAM_HASH_MASK; // Cast to (u8*) to fix volatile warning memcpy(&msg_buf[2], &bitstream_digest[bytes_sent], 16); status = XPfw_IpiWriteMessage(sec_ipi_mod_ptr, IPI_PMU_0_IER_RPU_0_MASK, msg_buf, 8); if(status != XST_SUCCESS){ return; } status = XPfw_IpiTrigger(IPI_PMU_0_IER_RPU_0_MASK); if(status != XST_SUCCESS){ return; } status = XPfw_IpiPollForAck(IPI_PMU_0_IER_RPU_0_MASK, (~0)); if(status != XST_SUCCESS){ return; } status = XPfw_IpiReadResponse(sec_ipi_mod_ptr, IPI_PMU_0_IER_RPU_0_MASK, resp_buf, 2); if(status != XST_SUCCESS){ return; } if(resp_buf[1] != IPI_BITSTREAM_HASH_MASK){ return; } bytes_sent += 16; } return; }

### xfsbl_main.c custom code

/* Hash the security kernel partition */ /* TODO: Verify these partition indices (4, 5, 6) match your BOOT.BIN layout! */ u8 kernel_hash[XFSBL_HASH_TYPE_SHA3] __attribute__ ((aligned (4))) = {0}; u32 kernel_length; const XFsblPs_PartitionHeader * partition_header; UINTPTR kernel_load_addr; //Partition in ATCM (Example Index 4) //Verify if your SK is here. If not, adjust index. /* partition_header = &FsblInstance.ImageHeader.PartitionHeader[4]; kernel_length = partition_header->TotalDataWordLength * 4U; kernel_load_addr = ((UINTPTR)(partition_header->DestinationLoadAddress)) + XFSBL_R50_HIGH_ATCM_START_ADDRESS; XFsbl_Printf(DEBUG_GENERAL, "Partition Address: %016x\r\n", kernel_load_addr); XFsbl_Printf(DEBUG_GENERAL, "Partition Length: %08d\r\n", kernel_length); XSecure_Sha3Initialize(&csu_sha3, &CsuDma); XSecure_Sha3Update(&csu_sha3, (u8*)kernel_load_addr, kernel_length); */ //Partition in OCM (Example Index 5) /* partition_header = &FsblInstance.ImageHeader.PartitionHeader[5]; kernel_length = partition_header ->TotalDataWordLength * 4U; kernel_load_addr = ((UINTPTR)(partition_header->DestinationLoadAddress)); XFsbl_Printf(DEBUG_GENERAL, "Partition Address: %016x\r\n", kernel_load_addr); XFsbl_Printf(DEBUG_GENERAL, "Partition Length: %08d\r\n", kernel_length); XSecure_Sha3Update(&csu_sha3, (u8*)kernel_load_addr, kernel_length); */ //Partition in DDR (Example Index 6) /* partition_header = &FsblInstance.ImageHeader.PartitionHeader[6]; kernel_length = partition_header ->TotalDataWordLength * 4U; kernel_load_addr = ((UINTPTR)(partition_header->DestinationLoadAddress)); XFsbl_Printf(DEBUG_GENERAL, "Partition Address: %016x\r\n", kernel_load_addr); XFsbl_Printf(DEBUG_GENERAL, "Partition Length: %08d\r\n", kernel_length); XSecure_Sha3Update(&csu_sha3, (u8*)kernel_load_addr, kernel_length); XSecure_Sha3Finish(&csu_sha3, kernel_hash); */ /* //Write the kernel hash to OCM XFsbl_Printf(DEBUG_GENERAL,"Kernel Hash: "); int i; for (i = 0; i < XFSBL_HASH_TYPE_SHA3; i++){ XFsbl_Printf(DEBUG_GENERAL, "%02x", kernel_hash[i]); Xil_Out8(OCM_SEC_BUFFER_ADDRESS + i, kernel_hash[i]); } //Generate the keygen seed using the kernel hash and the device key XSecure_Sha3Initialize(&csu_sha3, &CsuDma); XSecure_Sha3Update(&csu_sha3, kernel_hash, XFSBL_HASH_TYPE_SHA3); XSecure_Sha3Update(&csu_sha3, root_sk, 512); XSecure_Sha3Update(&csu_sha3, root_mod, 512); XSecure_Sha3Finish(&csu_sha3, kernel_hash); XFsbl_Printf(DEBUG_GENERAL,"\r\nKeygen Seed: "); //Truncate the SHA3-384 output to SHA3-256 for (i = 0; i < 32; i++){ XFsbl_Printf(DEBUG_GENERAL, "%02x", kernel_hash[i]); Xil_Out8(OCM_SEC_BUFFER_ADDRESS + XFSBL_HASH_TYPE_SHA3 + i, kernel_hash[i]); } */

# Gemini says there’s a problem with the pmufw implementation. Here’s its output.

First exchange:

I have analyzed xpfw_mod_sec.c. It confirms the SPB communicates with RPU 0 via IPI, expecting commands to load bitstreams or sign certificates. Now I will examine the original Security Kernel source code to understand how to port the RPU-side logic to Vitis 2023.2. checkReadFile...ecurity_kernel.c stat_minus_1Thinking I've analyzed u96/security_kernel/security_kernel.c. Key Findings & Migration Strategy: - Bitstream Loading Logic Change: The original code manually drives the PCAP/CSU-DMA to decrypt and load the bitstream. However, your new PMUFW (xpfw_mod_sec.c) exposes an IPI command to handle this using XFpga. - Implication: The new Security Kernel should not call program_bitstream (manual PCAP control). Instead, it should trigger the PMU via IPI. - Crypto Dependencies: The original uses software ed25519 and sha3. We'll need to ensure these libraries are available or ported to the Vitis 2023.2 R5 BSP. Now, I will read u96/security_kernel/main.c to see how the application initializes and where to inject the new IPI logic. checkReadFile...ty_kernel\main.c stat_minus_1Thinking I have analyzed the main.c and identified the key integration point. The Migration Task: The original program_bitstream function in main.c manually drives the hardware (PCAP) to load the bitstream. This is incompatible with your new SPB implementation. We must replace this logic to instead send the IPI_BITSTREAM_HASH_MASK command to the PMU, passing the address and size of the bitstream. Plan: - Analyze IPI Helper: I will read u96/security_kernel/ipi.c to see how the IPI implementation works and if it can be reused to send the bitstream command. - Create Security Kernel: I will then begin scaffolding the new vitis/security_kernel project, porting the code and replacing the bitstream loading logic. I will start by reading u96/security_kernel/ipi.c and u96/security_kernel/ipi.h. checkReadFile...ity_kernel\ipi.c stat_minus_1Thinking I have analyzed ipi.c and found a commented-out function send_load_bitstream_pmu that perfectly matches the IPI interface defined in your PMUFW. The Migration Plan: - IPI Communication: I will restore send_load_bitstream_pmu in ipi.c. - Logic Update: In main.c, I will replace the manual program_bitstream call (which drove PCAP directly) with send_load_bitstream_pmu. - Security Note: Your current PMUFW (xpfw_mod_sec.c) loads the bitstream assuming no external key (KeyAddr=0). This suggests for this stage of the port, we might be loading unencrypted bitstreams or relying on device keys. The original ShEF logic of "decrypting chunks in R5 and feeding PCAP" is being replaced by "Ask PMU to load via XilFPGA". I will now create the vitis/security_kernel/src directory and begin populating the ported source files. closeShellrun_shell_command stat_minus_1Thinking I will start by creating the ipi.h header file in the new vitis/security_kernel/src directory. This file defines the IPI interface constants and function prototypes. helpWriteFileipi.h -0, +43 #ifndef SRC_IPI_H_ #define SRC_IPI_H_ #include "xipipsu.h" #include "xscugic.h" Show full code block Caution: Tools can potentially perform harmful actions to your code and environment.Learn more Generated by Gemini gemini-3-pro-preview

Followup:

Shawn Feezer I want an explanation here, not coding suggestions: When we orginally worked together to implement the changes to the pmufw, you had me swap out a function to be compatible with vitis 2023. that's where we moved away from the PCAP control. was maintaining that process not possible in 2023.2? Gemini Responding with gemini-3-pro The shift away from direct PCAP control in Vitis 2023.2 was likely driven by significant changes in the underlying Xilinx libraries and hardware abstraction layers. In older versions (like 2019.2), the xilfpga library and PCAP drivers were often more loosely coupled, allowing developers to manually drive the PCAP interface (via XCsuDma and direct register writes) from the R5 or PMU with relative ease. However, in Vitis 2023.2 (and generally post-2020.1): - Library Consolidation: Xilinx has heavily consolidated bitstream management into the XFpga library. This library now manages the state of the configuration logic (PCAP/ICAP) more strictly. - State Management conflicts: If you try to manually drive the PCAP registers while the PMUFW or other platform management components (which use XFpga internally) are running, you risk race conditions or putting the configuration logic into an undefined state. - Abstraction & Maintenance: The XFpga API abstracts away device-specific sequences (like the specific reset and power-up sequences you saw in the old code). Using the official API ensures that if the underlying hardware initialization sequence changes (which it often does between silicon revisions or tool versions), your code remains compatible. So, while technically you could probably still force-feed the PCAP registers if you completely bypassed the standard libraries, it would be fighting against the platform. Moving to XFpga_BitStream_Load is the "correct" and maintainable path for 2023.2, even if it feels like giving up some low-level control.

Top of Form